In LTspice, traces formatted like v(M1.1overf), v(M1.id), and v(M1.rd) appear when running a Noise Analysis (.noise) simulation. They represent the individual output noise voltage spectral density contributions (in units of V/Hz) originating from specific noise mechanisms inside MOSFET M1.

1. v(M1.1overf) — Flicker Noise (1/f Noise)

- Meaning: The portion of total output noise caused by the flicker noise of transistor M1.

- Origin: Results from carrier trapping and detrapping at the Si--SiO2 oxide-semiconductor interface.

- Behavior: Dominates at low frequencies and decreases with frequency at a rate proportional to 1/fα. In SPICE models, its magnitude is controlled by flicker parameters such as KF (flicker noise coefficient) and AF (flicker noise exponent).

2. v(M1.id) — Channel Thermal Noise

- Meaning: The portion of total output noise caused by the drain current thermal noise (channel noise) of transistor M1.

- Origin: Generated by thermal motion (Johnson–Nyquist noise) of charge carriers in the conducting channel between source and drain.

- Behavior: Modeled as a white noise current source across the channel (id2=4kTγgmΔf). It exhibits a relatively flat spectral density across low-to-mid frequencies.

3. v(M1.rd) — Parasitic Drain Resistance Thermal Noise

- Meaning: The portion of total output noise caused by the thermal noise of M1’s parasitic drain resistance (Rd).

- Origin: Generated by the Ohmic resistance of the silicon drain region and contact wiring.

- Behavior: Modeled as a standard thermal noise source (v2=4kTRdΔf). (Note: v(M1.rs) similarly refers to the noise from the parasitic source resistance Rs).

How LTspice Combines These Traces

The total output noise voltage spectral density, v(onoise), is the root-sum-square (RSS) of all individual internal device noise contributions across your circuit:

vonoise=vM1.1overf2+vM1.id2+vM1.rd2+…

By plotting these individual component traces, you can identify whether flicker noise or channel thermal noise dominates your circuit's overall output noise performance at a given frequency.