(by Gemini AI):

In BSIM3v3 (SPICE3f5 Level 8, HSPICE Level 49, Version 3.1) for TSMC 0.18 µm (180 nm) CMOS processes, noise modeling is defined by two flicker (1/f) noise equations selectable via the NOIMOD parameter, alongside thermal noise formulations.

Below are the complete BSIM3v3 noise parameters for NMOS and PMOS, standard SPICE parameter syntax, and model card representations.

### BSIM3v3 Noise Model Parameters & Values

| Parameter | BSIM3 Keyword | Description | NMOS Value | PMOS Value |
| --- | --- | --- | --- | --- |
| Noise Model Selector | NOIMOD | 1 = Simple SPICE2/KF model 2 = BSIM3v3 Unified Noise Model | 2 (or 1) | 2 (or 1) |
| Flicker Noise Parameter A | NOIA | Unified flicker noise coefficient A (eV-1m-3) | 1.318×1019 | 3.575×1018 |
| Flicker Noise Parameter B | NOIB | Unified flicker noise coefficient B (eV-1m-1) | 1.445×105 | 2.500×103 |
| Flicker Noise Parameter C | NOIC | Unified flicker noise coefficient C (eV-1m) | -1.245×10-12 | 2.613×10-11 |
| Flicker Exponent | EF | Frequency exponent (1/fEF) | 0.92 to 1.0 | 1.14 |
| Saturation Field | EM | Saturation field parameter (V/m) | 4.10×107 | 4.10×107 |
| Flicker Noise Coeff (Simple) | KF | Empirical flicker noise coefficient (F⋅A) | 8.70×10-29 | 2.69×10-28 |
| Frequency Exponent (Simple) | AF | Frequency exponent for simple model | 1.0 | 1.0 |

### Standard SPICE 3f5 Level 8 / HSPICE Level 49 Parameter Deck

#### NMOS Model Card (CMOSN / N_180n)

Fragmento de código

.MODEL CMOSN NMOS (
+ LEVEL   = 49                  VERSION = 3.1           TNOM    = 27
+ TOX     = 4.1E-9              XJ      = 1E-7          NCH     = 2.3549E17
+ VTH0    = 0.3662              K1      = 0.5865        K2      = 1.127E-3
+ U0      = 273.81              UA      = -1.405E-9     UB      = 2.408E-18
+ UC      = 6.505E-11           VSAT    = 1.355E5       RDSW    = 123.3
+ LINT    = 1.69E-8             WINT    = 0.0           CGDO    = 8.23E-10
+ CGSO    = 8.23E-10            CJ      = 9.466E-4      CJSW    = 2.608E-10
+ PB      = 0.8                 MJ      = 0.382         PBSW    = 0.8
+ MJSW    = 0.102               CAPMOD  = 2             MOBMOD  = 1
* --- NOISE MODEL PARAMETERS ---
+ NOIMOD  = 2
+ NOIA    = 1.3182567E19        NOIB    = 1.4454397E5   NOIC    = -1.2451578E-12
+ EF      = 0.92                EM      = 4.1000000E7
* --- ALTERNATE SIMPLE NOISE MODEL (NOIMOD=1) ---
* + NOIMOD = 1                  KF      = 8.704E-29     AF      = 1.0
)

#### PMOS Model Card (CMOSP / P_180n)

Fragmento de código

.MODEL CMOSP PMOS (
+ LEVEL   = 49                  VERSION = 3.1           TNOM    = 27
+ TOX     = 4.1E-9              XJ      = 1E-7          NCH     = 4.1589E17
+ VTH0    = -0.3906             K1      = 0.5341        K2      = 0.0395
+ U0      = 115.69              UA      = 1.574E-9      UB      = 1.874E-21
+ UC      = -1.0E-10            VSAT    = 1.131E5       RDSW    = 198.3
+ LINT    = 2.945E-8            WINT    = 0.0           CGDO    = 6.35E-10
+ CGSO    = 6.35E-10            CJ      = 1.145E-3      CJSW    = 2.491E-10
+ PB      = 0.847               MJ      = 0.410         PBSW    = 0.877
+ MJSW    = 0.348               CAPMOD  = 2             MOBMOD  = 1
* --- NOISE MODEL PARAMETERS ---
+ NOIMOD  = 2
+ NOIA    = 3.5745699E18        NOIB    = 2.5000000E3   NOIC    = 2.6126002E-11
+ EF      = 1.1388              EM      = 4.1000000E7
* --- ALTERNATE SIMPLE NOISE MODEL (NOIMOD=1) ---
* + NOIMOD = 1                  KF      = 2.690E-28     AF      = 1.0
)

### BSIM3v3 Noise Model Formulations

#### 1. Unified Flicker Noise Model (NOIMOD = 2 or NOIMOD = 3)

Calculates flicker noise spectral density (Si,flicker) based on carrier number fluctuations and correlated mobility fluctuations:

Si,flicker=gm2⋅k⋅T⋅q2Weff⋅Leff⋅fEF⋅Cox2⋅NOIA⋅lnN0+N*Nl+N*+NOIB⋅(N0-Nl)+NOIC2(N02-Nl2)

where:

- N0 and Nl are the charge densities at the source and drain ends.

- N*=kT(Cox+Cd)/q2.

#### 2. Thermal Noise Model

Selected automatically depending on NOIMOD:

NOIMOD = 1 or 3: In long-channel MOSFET theory and legacy SPICE2 (or NOIMOD = 1 / NOIMOD = 3 in BSIM3v3), the drain-current thermal noise spectral density Si,thermal (A2/Hz) is expressed in terms of transconductance as:

Si,thermal=83kTgm

By substituting the relationship between transconductance (gm) and total channel inversion charge (Qchan), this equation converts directly into its charge-based equivalent:

Si,thermal=4⋅k⋅T⋅μL2Qchan

Step-by-Step Derivation

- Total Inversion Channel Charge (Qchan): In strong inversion and saturation for a long-channel MOSFET, the integrated total inversion charge under the gate is:

Qchan=23WLCoxVgs-Vth

- Transconductance (gm): The standard long-channel transconductance is defined as:

gm=μCoxWLVgs-Vth

- Connecting gm and Qchan: Solving for Vgs-Vth from Qchan gives Vgs-Vth=3Qchan2WLCox. Substituting this into gm:

gm=μCoxWL3Qchan2WLCox=3μ2L2Qchan

- Substitution into Thermal Noise: Substituting gm back into Si,thermal=83kTgm:

Si,thermal=83kT3μ2L2Qchan=4⋅k⋅T⋅μL2Qchan

- NOIMOD = 2 or 4: BSIM3v3 drain-current thermal noise density for short-channel model 
Si,thermal (A2/Hz) accounting for channel length modulation and velocity saturation:

Si,thermal=4⋅k⋅T⋅μeffLactive2∙Qinv

where:

- k: Boltzmann constant (1.380649×10-23 J/K)

- T: Absolute temperature (K)

- μeff: Effective mobility accounting for vertical electric field degradation

- Lactive: Active electrical channel length accounting for channel length modulation

- |Qinv|: Total integrated inversion layer charge in the channel

### Component Formulations

#### 1. Total Channel Inversion Charge (Qinv)

Rather than assuming a linear potential gradient along the channel, Qinv is integrated across the channel using a unified formulation valid in linear, saturation, and subthreshold regimes:

Qinv=Weff⋅Leff⋅Cox⋅Vgsteff-Abulk2Vdseff+Abulk2Vdseff212⋅Vgsteff

- Weff,Leff: Effective channel width and length.

- Cox: Gate oxide capacitance per unit area (Cox=εox/TOX).

- Vgsteff: Effective gate overdrive voltage (Vgs-Vth smoothly bounded above zero).

- Abulk: Bulk charge factor accounting for body effect along the channel.

- Vdseff: Smooth drain-source voltage function transitioning continuously into saturation (Vdsat).

#### 2. Channel Length Modulation (Lactive)

#### Channel Length Modulation (Lactive)

#### When Vds>Vdsat, the pinch-off point moves toward the source by a pinch-off region length ΔL. The active noise-generating length Lactive contracts:

#### Lactive=Leff-ΔL

#### ΔL=l⋅lnVds-Vdseffl⋅Esat+Vds-Vdseffl⋅Esat2+1

- l=εsiεox⋅TOX⋅Xj: Characteristic length of the velocity-saturated region (Xj is junction depth).

- Esat: Critical electric field for carrier velocity saturation (Esat=2⋅VSATμeff).

#### As Lactive shrinks in saturation, the 1/Lactive2 term causes local thermal noise to increase, accurately modeling short-channel thermal noise enhancement (excess noise factor γ>2/3).

#### 3. Carrier Velocity Saturation

Velocity saturation (vsat) limits the maximum drift velocity of carriers near the drain:

vd=μeffE1+E/Esat

### By incorporating Esat into the calculation of Vdsat and Vdseff, the spatial distribution of charge Qinvx reflects the field-dependent carrier density reduction across the channel.

### BSIM3v3 Thermal Noise Model Comparison

| Parameter / Feature | NOIMOD = 1 or 3 (SPICE2 Simple) | NOIMOD = 2 or 4 (BSIM3v3 Short-Channel) |
| --- | --- | --- |
| Formula | Si,thermal=4kTμL2Qchan=83kTgm | Si,thermal=4kTμeffLactive2¦Qinv¦ |
| Velocity Saturation | Ignored | Included via Vdseff and Esat |
| Channel Length Modulation | Ignored (ΔL=0) | Included via Lactive=Leff-ΔL |
| Excess Noise Ratio (γ) | Fixed at γ≈2/3 | Bias/length dependent (γ≥2/3, can reach 1.5-3.0) |
| Target Application | Legacy long-channel devices (L>1 μm) | Deep sub-micron processes (e.g., TSMC 0.18 μm) |